Date: Wed, 10 Apr 91 13:12:20 MDT

From: thorson@typhoon.atmos.colostate.edu (Bill Thorson)

__________=====

__________=====

__________=====    NSDSSO

Flexible  Image  Transport  System  (FITS)

December, 1990

Draft Standard

NSDSSO 100-0.1

NASA Science Data Systems Standards Office

Code 933

NASA Goddard Space Flight Center

Greenbelt MD 20771

The NASA Science Data Systems Standards Office (NSDSSO) has been

established to serve the space science communities in evolving cost effective,

interoperable, data systems. The NSDSSO performs a number of functions designed

to facilitate the recognition, development, adoption, and use of standards by

the space science communities.

Approval of an NSDSSO Standard requires verification by the NSDSSO that the

following requirements have been met: consensus by the technical panel; proper

adjudication of the comments received from the targeted space and Earth

science community; and conformance to the accreditation process.

An NSDSSO standard represents the consensus of the technical panel convened

by the NASA Science Data Systems Standards Office (NSDSSO) of the National

Space Science Data Center (NSSDC) of the National Aeronautics and Space

Administration (NASA). Consensus is established when the NSDSSO Accreditation

Panel determines that substantial agreement has been reached by the Technical

Panel. However consensus does not necessarily imply that all members were in

full agreement with every item in the standard. NSDSSO standards are not

binding as published, however they may serve as a basis for mandatory standards

when adopted by NASA or other organizations.

An NSDSSO standard may be revised at any time depending on developments in

the areas covered by the standard. Also, within five years from the date of its

issuance, this standard will be reviewed by the NSDSSO to determine whether it

should 1) remain in effect without change, 2) be changed to reflect the impact

of new technologies or new requirements, or 3) be retired or cancelled.

The Technical Panel which developed this standard consisted of the

following members:

Robert J. Hanisch, Chair           Space Telescope Science Institute

Barry M. Schlesinger, Secretary    ST Systems Corporation

Lee E. Brotzman                    ST Systems Corporation

Edward Kemper                      ST Systems Corporation

Peter Teuben                       University of Maryland

Michael E. Van Steenberg           NASA Goddard Space Flight Center

Wayne H. Warren Jr.                ST Systems Corporation

Richard A. White                   NASA Goddard Space Flight Center

This standard is published and maintained by the NSDSSO. Send comments and

orders for NSDSSO documents to:

NSDSSO, Code 933, NASA Goddard Space Flight Center

Greenbelt MD 20771

Internet:  NSDSSO@nssdca.gsfc.nasa.gov

SPAN: nssdca::NSDSSO

301-286-3575

CONTENTS_____________________________________________________________________i

Contents

Introduction                                                                v

1    Overview                                                               1

1.1    Purpose  ...................................................    1

1.2    Scope  .....................................................    1

1.3    Applicability   ............................................    1

1.4    Organization and Recommendations   .........................    2

2    References                                                             3

3    Definitions, Acronyms, and Symbols                                     5

4    Data Set Organization                                                  9

4.1    Overall Structure  .........................................    9

4.2    FITS Element Structure  ....................................    9

4.3    Primary Header and Data  ...................................    9

4.3.1     Primary Header    ................................   10

4.3.2     Primary Data  ....................................   10

4.4    Extensions  ................................................   10

4.4.1     Requirements for Conforming Extensions   .........   10

4.4.2     Standard Extensions  .............................   11

4.4.3     Order of Extensions  .............................   11

4.5    Special Records  ...........................................   11

5    Headers                                                               13

5.1    Card Images    .............................................   13

5.1.1     Syntax   .........................................   13

5.1.2     Components    ....................................   13

5.2    Keywords   .................................................   14

5.2.1     Mandatory Keywords   .............................   14

5.2.2     Other Reserved Keywords  .........................   18

5.2.3     Additional Keywords  .............................   23

5.3     Value/Comment   ............................................   24

5.3.1     Character String   ...............................   24

5.3.2     Logical Variable   ...............................   24

5.3.3     Integer  .........................................   24

5.3.4     Real Floating Point Number   .....................   24

5.3.5     Complex Floating Point Number  ...................   25

6   Data Representation                                                    27

6.1     Characters  ................................................   27

6.2     Integers   .................................................   27

6.2.1     Eight-bit    .....................................   27

6.2.2     Sixteen-bit  .....................................   27

6.2.3     Thirty-two-bit   .................................   27

_______________________________________________________________________________

NSDSSO FITS Standard

ii_____________________________________________________________________CONTENTS

6.3     IEEE-754 Floating Point   ..................................   28

6.3.1     Thirty-two-bit Floating Point  ...................   28

6.3.2     Sixty-four-bit Floating Point  ...................   29

7   Data Records                                                           31

7.1     Primary Array   ............................................   31

7.2     ASCII Tables Extension  ....................................   31

7.2.1     Storage   ........................................   31

7.2.2     Fields   .........................................   33

7.2.3     Entries  .........................................   33

8   Restrictions on Changes                                                35

Appendixes

A   Random Groups Structure                                                37

A.1     Keywords   .................................................   37

A.1.1     Mandatory Keywords   .............................   37

A.1.2     Reserved Keywords  ...............................   39

A.2     Data Storage   .............................................   40

A.3     Data Representation   ......................................   40

B   A Binary Table Extension                                               41

B.1     Identification   ...........................................   41

B.2     Header   ...................................................   41

B.2.1     Required Keywords  ...............................   41

B.2.2     Reserved Keywords   ..............................   44

B.3    Storage   ..................................................   44

B.4    Format   ...................................................   44

B.5    Data Types   ...............................................   45

B.5.1     Logical   ........................................   45

B.5.2     Bit Arrays   .....................................   45

B.5.3     ASCII Characters    ..............................   45

B.5.4     Eight-bit Integers   .............................   45

B.5.5     Sixteen-bit Integers   ...........................   45

B.5.6     Thirty-two-bit Integers   ........................   45

B.5.7     Thirty-two-bit Floating Point   ..................   46

B.5.8     Sixty-four-bit Floating Point   ..................   46

C    Implementation on Physical Media                                      47

C.1    Block Size   ...............................................   47

C.1.1     Magnetic Tape   ..................................   47

C.1.2     Other Media   ....................................   47

C.2    Physical Properties of Media   .............................   48

C.3    Labeling   .................................................   48

C.3.1     Tape   ...........................................   48

_______________________________________________________________________________

NASA Science Data Systems Standards Office

List_of_Tables______________________________________________________________iii

C.3.2     Other Media   ....................................   48

C.4    FITS File Boundaries   .....................................   48

C.4.1     Magnetic Tape   ..................................   48

C.4.2     Other Media   ....................................   48

C.5    Multiple Physical Volumes   ................................   48

D    Summary of Keywords                                                   49

E    NSDSSO Publications                                                   51

List  of  Tables

5.1    Principal mandatory keywords   .............................   14

5.2    Interpretation of BITPIX value   ...........................   15

5.3    Mandatory keywords in conforming extensions   ..............   16

5.4    Mandatory keywords in ASCII tables extensions   ............   17

5.5    Valid TFORMn format values in TABLE extensions   ...........   18

6.1    Content of 32-bit floating point bit positions   ...........   28

6.2    Content of 64-bit floating point bit positions   ...........   29

B.1    Required keywords in the interim binary table extension   ..   42

B.2    Valid TFORMn format values in the interim binary table

extension     ..............................................   43

D.1    Mandatory FITS keywords   ..................................   49

D.2    Reserved FITS keywords   ...................................   50

D.3    General Reserved FITS keywords   ...........................   50

E.1    NSDSSO Publications   ......................................   51

List  of  Figures

7.1     Array data storage sequence   ..............................   32

_______________________________________________________________________________

NSDSSO FITS Standard

iv_____________________________________________________________________CONTENTS

_______________________________________________________________________________

NASA Science Data Systems Standards Office

_____________________________________________________________________________v

Introduction

The Flexible Image Transport System (FITS) evolved out of the recognition that

a standard format was needed for transferring astronomical data from one

installation to another. The original form, or Basic FITS, was designed for the

transfer of images and consisted of a binary array, usually multidimensional,

preceded by an ASCII text header with information describing the structure and

contents of the array. The FITS concept was later expanded to accommodate more

complex data structures. For example, a new structure for image transfer called

random groups was defined in which the data consists of a series of arrays,

with each array accompanied by a set of associated parameters. These structures

were formally endorsed by the International Astronomical Union (IAU) in

1982[1]. Provisions for data structures other than simple arrays or groups were

later made. These structures appear in extensions, consisting of an ASCII

header followed by the data structure it describes. One particular extension

structure, ASCII Tables, was formally approved by the IAU in 1988. At the same

time, a set of rules governing all extensions was endorsed, to ensure that FITS

readers encountering an unfamiliar extension would be able to identify it,

interpret it, and pass on to the next.

FITS was originally designed and defined for 9-track half-inch magnetic

tape. However, as improvements in technology have brought forward other data

storage and data distribution media, it has generally been agreed that the FITS

structure is to be understood as a logical structure, and not defined in terms

of the physical characteristics of any particular data storage medium or media.

_______________________________________________________________________________

NSDSSO FITS Standard

vi_____________________________________________________________________CONTENTS

_______________________________________________________________________________

NASA Science Data Systems Standards Office

______________________________________________________________________________1

Section   1

Overview

1.1        Purpose

This standard formally defines the FITS format for data structuring and

exchange. It is intended as a formal codification of the FITS structure that

has been endorsed by the IAU for transfer of astronomical data, as described in

four published papers [2,3,4,5] and in a formal agreement [6] endorsed by the

IAU FITS Working Group under Commission 5. Minor ambiguities and

inconsistencies in FITS as described in the original papers are eliminated.

1.2        Scope

This standard specifies the structure and content of FITS data sets. It

specifies the content and organization of the header and data for all standard

FITS formats: Basic FITS, the ASCII tables extension, and the random groups

structure. It also specifies minimum structural requirements for new extensions

and general principles governing the creation of new extensions. For headers,

it specifies the proper syntax for card images and defines required and

reserved keywords. For data, it specifies character and value representations.

It defines the general rules to which new extensions are required to conform.

1.3        Applicability

All spacecraft projects and astrophysics data archives under the management of

the Astrophysics Division of the National Aeronautics and Space Administration

are required to make processed data available to users in the format defined by

this standard, unless the Astrophysics Division specifically determines

otherwise. The IAU has recommended that all astronomical computer facilities

support FITS for the interchange of binary data.  This standard may also be

used

_______________________________________________________________________________

NSDSSO FITS Standard

2_________________________________________________________SECTION_1.___OVERVIEW

used to define the format for data transport in other disciplines, as may be

determined by the appropriate authorities.

1.4        Organization  and  Recommendations

Following the definitions in Section 3, this document describes the overall

organization of a FITS data set and the rules for creating new FITS extensions

in Section 4. Subsequent sections describe the contents of FITS files. Section

5 describes headers, including the required keywords and the conditions under

which each is required; also, reserved keywords are specified. Section 6

describes how data are represented, and Section 7 describes data structures.

Each section contains information relevant to all FITS formats. Throughout the

document, deprecation of structures or syntax is noted where relevant. Data

sets containing such constructs are valid FITS, but these constructs should not

be used in new data sets; the old data sets using them remain standard because

of the principle that no FITS structure, once valid, shall ever become invalid.

Although it is part of the FITS standard, the random groups structure has

not been widely used. New formats under development will replace it, and use of

it is discouraged. Consequently, it is described in Appendix A rather than in

the main body of the standard. One new format, 3D binary tables, is described

for informational purposes in Appendix B, which is not part of the formal

standard.

While FITS is designed as a logical format, the astronomical community has

developed recommendations for its expression on various physical media. The

accepted recommendations are provided in Appendix C as a guide to FITS

practices, but they are not part of this standard. Appendix D gives a tabular

summary of the FITS keywords.

_______________________________________________________________________________

NASA Science Data Systems Standards Office

______________________________________________________________________________3

Section   2

References

1.  IAU Information Bulletin No. 49, p. 14, 1983.

2.  Wells, D. C., Greisen, E. W., and Harten, R. H. 1981, "FITS: A Flexible

Image Transport System," Astron. Astrophys. Suppl., 44, 363-370.

3.  Greisen, E. W. and Harten, R. H. 1981, "An Extension of FITS for Small

Arrays of Data," Astron. Astrophys. Suppl., 44, 371-374.

4.  Grosbol, P., Harten, R. H., Greisen, E. W., and Wells, D. C. 1988,

"Generalized Extensions and Blocking Factors for FITS," Astron.

Astrophys. Suppl., 73, 359-364.

5.  Harten, R. H., Grosbol, P., Greisen, E. W., and Wells, D. C. 1988, "The

FITS Tables Extension," Astron. Astrophys. Suppl., 73, 365-372.

6.  Wells, D. C. and Grosbol, P. 1990, "Floating Point Agreement for FITS"

(available from the authors at National Radio Astronomy Observatory,

Edgemont Road Charlottesville, Virginia 22903 USA or European Southern

Observatory Karl-Schwarzschild Strasse 2, D-8046 Garching bei Munchen,

FRG.)

7.  "American National Standard for Information Processing: Programming

Language FORTRAN," ANSI X3.9 - 1978 (ISO 1539). Published by American

National Standards Institute, Inc., New York, 1978.

8.  "American National Standard for Information Processing: Code for

Information Interchange," ANSI X3.4 - 1977 (ISO 646). Published by

American National Standards Institute, Inc., New York, 1977.

9.  "American National Standard - IEEE Standard for Binary Floating Point

Arithmetic". ANSI/IEEE 754-1985, Published by American National

Standards Institute, Inc., New York, 1985.

_______________________________________________________________________________

NSDSSO FITS Standard

4_______________________________________________________SECTION_2.___REFERENCES

10.   "American National Standard for Information Processing: Unrecorded

Magnetic Tape," ANSI X3.40 - 1976, Published by American National

Standards Institute, Inc., New York, 1976.

11.   "American National Standard for Information Processing: Magnetic Tape

Labels and File Structure," ANSI X3.27 - 1978, Published by American

National Standards Institute, Inc., New York, 1978.

_______________________________________________________________________________

NASA Science Data Systems Standards Office

______________________________________________________________________________5

Section   3

Definitions, Acronyms, and Symbols

~ (tilda)     Used to designate an ASCII blank.

Array          A sequence of data values, of zero or more dimensions.

Array value    An element of an array.

ASCII          Abbreviation of American National Standard Code for Information

Interchange.

Basic FITS     The FITS structure consisting of the primary header and primary

data containing a single array.

Bit            A single binary digit.

Byte           A string of eight bits treated as a single entity.

Card image     A sequence of 80 bytes with values between hexadecimal 20 and 7E

inclusive, treated as a logical record.

Conforming extension    An extension whose keywords and structure adhere to the

requirements for conforming extensions defined in this standard.

Deprecate      To express earnest disapproval of (permitted but discouraged).

FITS Element   One of the components of a FITS data set: the primary HDU, an

extension, or the special records collectively.

Entry          A single value in a table.

Extension      A FITS HDU following the primary HDU in a FITS file.

_______________________________________________________________________________

NSDSSO FITS Standard

6_______________________________SECTION_3.___DEFINITIONS,_ACRONYMS,_AND_SYMBOLS

Field          A set of zero or more table entries collectively described by a

single format.

File           A sequence of one or more records terminated by an end-of-file

indicator.

FITS           Abbreviation of Flexible Image Transport System.

FITS file      A file with a structure that matches the specifications in this

document.

FITS logical record      A record of 23040 bits, corresponding to 2880 8-bit

bytes within a FITS file.

Floating point      A number stored internally as a mantissa and exponent,

whose ASCII representation contains an explicit decimal point

and may include a power-of-ten exponent.

Group Format   One of two possible data organization formats for the primary

HDU.

Header         A series of card images organized within one or more FITS

Logical Records which describes structures and/or data which

follow it in the FITS file.

Header and Data Unit (HDU)    A data structure consisting of a Header and the

Data structure the Header describes.  Note that an HDU may

consist entirely of a header with no data records.

IAU            Abbreviation of International Astronomical Union.

IEEE           Abbreviation of Institute of Electrical and Electronic

Engineers.

IEEE NaN       Abbreviation of IEEE Not-a-Number value.

_______________________________________________________________________________

NASA Science Data Systems Standards Office

______________________________________________________________________________7

IEEE special values      (-0, +/- infinity).

Keyword        The first eight bytes of a header card image.

Mandatory keyword   A keyword which must be used in all FITS data sets or a

keyword required in conjunction with particular FITS structures.

Matrix         A data array of two or more dimensions.

NSDSSO         Abbreviation of NASA Science Data Systems Standards Office.

Physical value      The value in physical units represented by a member of an

array and possibly derived from the array value using a linear

transformation.

Picture element     A single location within an image array.

Pixel          Abbreviation of "picture element".

Primary array       The data array contained in the Primary HDU.

Primary header      The first header in a FITS file, containing information on

the overall structure of the file as well as on the primary

array.

Random Group        A primary HDU data structure, which consists of a sequence

of individual values followed by a data array.

Random Group parameters       The sequence of individual values that precede

the data array in a Random Group.

Record         A sequence of bits treated as a single logical entity.

_______________________________________________________________________________

NSDSSO FITS Standard

8_______________________________SECTION_3.___DEFINITIONS,_ACRONYMS,_AND_SYMBOLS

Reference pixel     The pixel along a given coordinate axis at which a value

and increment are defined.

Reserved keyword    An optional keyword that may be used only in the manner

defined in this standard.

Special Records     A series of 23040-bit (2880 8-bit byte) records, at the end

of the FITS file, whose internal structure does not otherwise

conform to that for the primary HDU or to that specified for a

conforming extension in this standard.

Standard Extension       A conforming extension whose header and data content

are specified explicitly in this standard.

Valid value    A member of a data array or table corresponding to an actual

physical quantity.

_______________________________________________________________________________

NASA Science Data Systems Standards Office

______________________________________________________________________________9

Section   4

Data   Set   Organization

4.1        Overall  Structure

A FITS file shall be composed of the following FITS elements, in the order

listed:

o  Primary HDU

o  Extensions (optional)

o  Special records (optional)

Each FITS element shall consist of an integral number of FITS logical

records. The primary HDU shall start with the first record of the FITS file.

The first record of each subsequent FITS element shall be the record

immediately following the last record of the preceding FITS element. The size

of a FITS logical record shall be 23040 bits, corresponding to 2880 8-bit

bytes.

4.2        FITS  Element  Structure

The primary HDU and every extension shall consist of an integral number of

header records consisting of ASCII text, which may be followed by an integral

number of data records. The first record of data shall be the record

immediately following the last record of the header.

4.3        Primary  Header  and  Data

The first component of a FITS data set shall be the primary header. The primary

header may, but need not be, followed by primary data. The presence or absence

of primary data shall be indicated by the value of the NAXIS keyword in the

primary header (Section 5.2.1.1).

_______________________________________________________________________________

NSDSSO FITS Standard

10___________________________________________SECTION_4.___DATA_SET_ORGANIZATION

4.3.1       Primary  Header

The header of a primary HDU shall consist of a series of card images in ASCII

code. All header records shall consist of 36 card images. Card images without

information shall be filled with ASCII blanks (hexadecimal 20).

4.3.2       Primary  Data

The primary data, if present, may be either in Basic FITS array format or

random groups format. In Basic FITS format, the primary data shall consist of a

single data array of 1-999 dimensions. If the data array does not fill the

final record, the remaining storage locations shall be filled with zero values

with the same data representation as the values in the array. Random groups

format, which this document deprecates, is described in Appendix A.

4.4        Extensions

All extensions, whether or not described in this standard, shall fulfill the

following requirements to be in conformance with this FITS standard.

4.4.1       Requirements  for  Conforming  Extensions

4.4.1.1       Identity

Each extension shall have a unique name, specified in the header according to

the syntax codified in Section 5.2.1.2. To preclude conflict, extension names

must be registered with the NSDSSO or IAU Commission 5 who will coordinate name

allocations.

4.4.1.2       Size Specification

The total number of bits in the data of each extension shall be specified in

the header for that extension, in the manner prescribed in Section 5.2.1.2.

_______________________________________________________________________________

NASA Science Data Systems Standards Office

4.5.___SPECIAL_RECORDS_______________________________________________________11

4.4.1.3       Compatibility with Existing FITS Structures

No extension shall be constructed that invalidates existing FITS data sets.

4.4.2       Standard  Extensions

A standard extension shall be a conforming extension whose structure and

content are completely specified in this standard. Only one FITS format shall

be approved for each type of data organization. Each standard extension shall

have a unique name.

4.4.3       Order  of  Extensions

Standard extensions and other conforming extensions may appear in any order in

a FITS data set.

4.5        Special  Records

The first 8 bytes of special records must not contain the string "XTENSION" or

the string "SIMPLE  ". The records must have the standard FITS 23040-bit record

length. The contents of special records are not otherwise specified by this

standard.

_______________________________________________________________________________

NSDSSO FITS Standard

12___________________________________________SECTION_4.___DATA_SET_ORGANIZATION

______________________________________________________________________________

NASA Science Data Systems Standards Office

____________________________________________________________________________13

Section   5

Headers

5.1        Card  Images

5.1.1       Syntax

Header card images shall consist of a keyword, an optional value, and an

optional comment. If a value is present, column 9 shall contain the symbol "=",

column 10 shall be blank, and columns 11-80 shall be as specified in the

remainder of Section 5.2. If no value is present, column 9 may contain any

ASCII text except the symbol "=", and columns 10-80 may contain any ASCII text.

5.1.2       Components

5.1.2.1       Keyword (bytes 1-8)

The keyword shall be a left justified, 8-character, blank filled, ASCII string

with no embedded blanks. All digits (hexadecimal 30 to 39, "0123456789") and

upper case Latin alphabetic characters (hexadecimal 41 to 5A, "ABCDEFG HIJKLMN

OPQRST UVWXYZ") are permitted; no lower case characters shall be used. The

underscore (hexadecimal 60, "_") and hyphen (hexadecimal 2D, "-") are also

permitted. No other characters are permitted. If the keyword itself requires

less than eight characters it must be filled with ASCII blanks (hexadecimal

20). The value and comments field on a card image may be restricted for

particular keywords.

5.1.2.2       Value Indicator (bytes 9-10)

This field shall contain an ASCII "= " for keywords with an associated value

field. If there is no associated value field, byte 9 may not contain "="

(hexadecimal 3D); otherwise, this field may contain any ASCII character text.

_______________________________________________________________________________

NSDSSO FITS Standard

14_________________________________________________________SECTION_5.___HEADERS

5.1.2.3       Value/Comment

This field, when used, shall contain the value, if any, of the keyword followed

by optional comments. Separation of the value and comments by a slash

(hexadecimal 2F, "/"), and a space between the value and the slash are strongly

recommended. The value shall be the ASCII representation of a string or

constant, structured as specified in Section 5.3. The comment may contain any

printable 7-bit ASCII characters (hexadecimal 20 through 7E).

5.2        Keywords

5.2.1       Mandatory  Keywords

Mandatory keywords are required as described in the remainder of this

subsection. They may be used only as described in this standard.

5.2.1.1       Principal

Principal Mandatory keywords other than SIMPLE are required in all FITS

headers. The SIMPLE keyword is required in all primary headers and may not

appear elsewhere. The card images of any primary header must contain the

keywords shown in Table 5.1 in the order given.

1      SIMPLE

2      BITPIX

3      NAXIS

4      NAXISn, n = 1, . . ., NAXIS

.

.

(other keywords)

.

.

last    END

Table 5.1:  Principal mandatory keywords.

SIMPLE Keyword     The value field shall contain a logical constant with the

value T if the file conforms to this standard. This keyword is mandatory for

the primary header and must not appear as the first keyword in a conforming

extension header.

_______________________________________________________________________________

NASA Science Data Systems Standards Office

5.2.___KEYWORDS______________________________________________________________15

BITPIX Keyword     The value field shall contain an integer. The absolute value

is used in computing the sizes of data structures. It shall specify the number

of bits that represent a data value (see Table 5.2).

__Value____________Data_Represented______________

8    Character or unsigned binary integer

16    16-bit twos complement binary integer

32    32-bit twos complement binary integer

-32    IEEE single precision floating point

-64    IEEE double precision floating point

Table 5.2:  Interpretation of BITPIX value.

NAXIS Keyword      The value field shall contain an integer no greater than

999, representing the number of axes in an ordinary data array. A value of zero

signifies that no data follow the header.

NAXISn  Keywords   The value field shall contain an integer, representing the

number of positions along axis n of an ordinary data array. If NAXIS is equal

to 0, there should not be any NAXISn keywords.

END Keyword    This keyword has no associated value. Columns 9-80 shall be

blank.

5.2.1.2       Conforming Extensions

The use of FITS extensions requires the use of a single additional keyword in

the primary header.

EXTEND Keyword     If the FITS data set contains extensions, a card image with

the keyword EXTEND and the value field containing the logical value T must

appear in the primary header immediately after the last NAXISn card image, or,

if NAXIS=0, the NAXIS card image. The presence of this keyword with the value T

in the primary header does not require that extensions be present.

The card images of any extension header must use the keywords defined in

Table 5.3 in the order specified. This organization is required for any

conforming extension, whether or not further specified in this standard.

The total number of bits in the extension data array exclusive of the fill

described in Section 4.3.2 must be given by the following expression:

_______________________________________________________________________________

NSDSSO FITS Standard

16_________________________________________________________SECTION_5.___HEADERS

1      XTENSION

2      BITPIX

3      NAXIS

4      NAXISn, n = 1, . . ., NAXIS

.

.

(other keywords, including . . .)

PCOUNT

GCOUNT

.

.

last      END

Table 5.3:  Mandatory keywords in conforming extensions.

NBITS = |BITPIX| x GCOUNT  x

(PCOUNT + NAXIS1 x NAXIS2 x ... x NAXISm);

where NBITS is the number of bits excluding fill, m is the value of NAXIS, and

BITPIX, GCOUNT, PCOUNT, and the NAXISn represent the values associated with

those keywords.

XTENSION  Keyword  The value field shall contain a character string giving the

name of the extension type. This keyword is mandatory for an extension header

and must not appear in the primary header. For an extension that is not a

standard extension, the type name must not be the same as that of a standard

extension. NSDSSO, in collaboration with IAU Commission 5, may specify

additional names that must be used only to identify specific types of

extensions.

PCOUNT  Keyword    The value field shall contain an integer that shall be used

in any way appropriate to define the data structure, consistent with the

equation for NBITS. Usage is analogous to that of the PCOUNT keyword for the

Random Groups structure (Section A.1.1), which represents the number of

parameters in a parameter-array group.

GCOUNT  Keyword    The value field shall contain an integer that shall be used

in any way appropriate to define the data structure, consistent with the

equation for NBITS. Usage is analogous to that of the GCOUNT keyword for the

Random Groups structure (Section A.1.1), which represents the number of

parameter-array groups.

END Keyword    This keyword has no associated value.  Columns 9-80 shall be

blank.

_______________________________________________________________________________

NASA Science Data Systems Standards Office

5.2.___KEYWORDS______________________________________________________________17

5.2.1.3       ASCII Tables Extension Keywords

The card images in the header of an ASCII Tables Extension must use the

keywords defined in Table 5.4 in the order specified.

1      XTENSION

2      BITPIX

3      NAXIS

4      NAXIS1

5      NAXIS2

6      PCOUNT

7      GCOUNT

8      TFIELDS

..

.

(other keywords, which must include . . .)

TBCOLn, n=1,2,. . .,k where k is the value of TFIELDS

TFORMn, n=1,2,. . .,k where k is the value of TFIELDS

..

.

last     END

Table 5.4:  Mandatory keywords in ASCII tables extensions.

XTENSION Keyword   The value field shall contain the character string

'TABLE~~~'.

BITPIX Keyword     The value field shall contain the integer 8, denoting that

the array contains 8-bit printable ASCII characters with the eighth bit, the

parity bit, set to zero (i.e. hexadecimal codes 20 through 7E).

NAXIS Keyword      The value field shall contain the integer 2, denoting that

the included data array is two-dimensional: rows and columns.

NAXIS1  Keyword    The value field shall contain an integer, representing the

width of the table in characters.

NAXIS2  Keyword    The value field shall contain an integer of value 1 or

greater, representing the number of rows in the table.

_______________________________________________________________________________

NSDSSO FITS Standard

18_________________________________________________________SECTION_5.___HEADERS

PCOUNT  Keyword    The value field shall contain the integer 0; no data values

precede the table.

GCOUNT Keyword     The value field shall contain the integer 1; the data

records contain a single table.

TFIELDS  Keyword   The value field shall contain an integer representing the

number of fields in each row. The maximum permissible value is 999.

TBCOLn  Keywords   The value field shall contain an integer specifying the

column in which field n starts.

TFORMn  Keywords   The value field shall contain a character string describing

the FORTRAN-77 [7] format in which field n is coded. The formats in Table 5.5

may be used for encoding.

_Field_Value_________Data_Type_____________________________

Aw        Character

Iw        Integer

Fw.d      Real

Ew.d      Single precision real, exponential notation

Dw.d      Double precision real, exponential notation

Table 5.5:  Valid TFORMn format values in TABLE extensions.

Repetition of a format from one field to the next must be indicated by

using separate pairs of TBCOLn and TFORMn keywords for each field; format

repetition may not be indicated by prefixing the format by a number.

END Keyword    This keyword has no associated value.  Columns 9-80 shall be

blank.

5.2.2       Other  Reserved  Keywords

These keywords are optional but may be used only as defined in this standard.

These keywords apply to any FITS structure except where specifically further

restricted.

_______________________________________________________________________________

NASA Science Data Systems Standards Office

5.2.___KEYWORDS______________________________________________________________19

5.2.2.1     Keywords Describing the History or Physical Construction of the HDU

DATE  Keyword      The value field shall contain a character string giving the

date on which the HDU was created, in the form DD/MM/YY, where DD shall be the

day of the month, MM the month number, with January given by 01 and December by

12, and YY the last two digits of the year. For all data sets created after the

adoption of this standard, the date shall be that specified by local civil time

at the location where the data set was created. Copying of a FITS file does not

require changing any of the keyword values in the file's HDUs.

ORIGIN  Keyword    The value field shall contain a character string identifying

the organization creating the data set.

BLOCKED  Keyword   This keyword may be used only in the primary header. It must

appear immediately after the EXTEND card image, or if this card image is not

present, after the last NAXISn card image or the NAXIS card image with value

zero. Its presence with the required logical value of T advises that the

physical block size of the FITS data set on which it appears may be an integral

multiple of the logical record length. Physical block size and logical record

length may be equal even if this keyword is present or unequal if it is absent.

It is reserved primarily to prevent its use with other meanings. The issuance

of this standard deprecates the BLOCKED keyword.

5.2.2.2       Keywords Describing Observations

DATE-OBS Keyword   The value field shall contain a character string giving the

day on which the observations represented by the array were made, in Universal

Time in the form form DD/MM/YY, where DD shall be the day of the month, MM the

month number, with January given by 01 and December by 12, and YY the last two

digits of the year.

TELESCOP  Keyword  The value field shall contain a character string identifying

the telescope used to acquire the data contained in the array.

INSTRUME  Keyword  The value field shall contain a character string identifying

the instrument used to acquire the data contained in the array.

OBSERVER Keyword   The value field shall contain a character string identifying

the individual who acquired the data associated with the header. This keyword

is appropriate when the data describe the results of observations.

_______________________________________________________________________________

NSDSSO FITS Standard

20_________________________________________________________SECTION_5.___HEADERS

OBJECT Keyword     The value field shall contain a character string giving the

name of the object observed.

EQUINOX Keyword    The value field shall contain a character string giving the

equinox for the celestial coordinate system in which positions given in either

the header or data are expressed.

EPOCH  Keyword     The value field shall contain a character string giving the

equinox for the celestial coordinate system in which positions given in either

the header or data are expressed. This document deprecates the use of the EPOCH

keyword and thus it shall not be used in data sets created after the adoption

of this standard; rather, the EQUINOX keyword shall be used.

5.2.2.3       Bibliographic Keywords

AUTHOR  Keyword    The value field shall contain a character string identifying

the individual who compiled the information in the data associated with the

header. This keyword is appropriate when the data originate in a published

paper or are a compilation of results from many sources.

REFERENC Keyword   The value field shall contain a character string citing a

reference where the data associated with the header are published.

5.2.2.4       Commentary Keywords

COMMENT  Keyword   This keyword shall have no associated value; columns 9-80

may contain any ASCII text, except that column 9 may not contain "=". Any

number of COMMENT records may appear in a header.

HISTORY  Keyword   This keyword shall have no associated value; columns 9-80

may contain any ASCII text, except that column 9 may not contain "=". The text

should contain a history of steps and procedures associated with the processing

of the associated data. Any number of HISTORY records may appear in a header.

Keyword Field is Blank   Columns 1-8 are blank. Columns 9-80 may contain any

ASCII text except that column 9 may not contain "=", as for a keyword with no

associated value. Any number of blank records may appear in a header.

_______________________________________________________________________________

NASA Science Data Systems Standards Office

5.2.___KEYWORDS______________________________________________________________21

5.2.2.5       Array Keywords

These keywords are used to describe the contents of a simple array, either

alone, or in a series of random groups. They are optional, but if they appear

in the header describing an array, they must be used as defined in this section

of this standard. They shall not be used in headers describing other structures

unless the meaning is the same as that for a primary array.

BSCALE  Keyword    This keyword shall be used, along with the BZERO keyword,

when the array pixel values are not the true physical values, to transform the

primary array values to the true physical values they represent, using the

linear transformation equation defined below. The value field shall contain a

floating point number representing the coefficient of the linear term in the

scaling equation, the ratio of physical value to array value at zero offset.

BZERO  Keyword     This keyword shall be used, along with the BSCALE keyword,

when the array pixel values are not the true physical values, to transform the

primary array values to the true values. The value field shall contain a

floating point number representing the physical value corresponding to an array

value of zero. The transformation equation is as follows:

physical value = BZERO + BSCALE x array value

BUNIT  Keyword     The value field shall contain a character string, describing

the ultimate physical units in which the quantities of the array are expressed.

BLANK  Keyword     This keyword shall be used only in headers with positive

values of BITPIX (i.e. in arrays with integer data). Columns 1-8 contain the

string, "BLANK~~~" (blanks in columns 6-8). The value field shall contain an

integer that specifies the representation of array members whose data values

are undefined.

CTYPEn Keywords    The value field shall contain a character string, giving the

physical coordinate represented by axis n.

CRPIXn  Keywords   The value field shall contain a floating point number,

identifying the location of a reference point along axis n, in units of the

axis index. The reference point need not coincide with the center of a pixel

(i.e. need not be an integer), nor lie within the actual data array.

CRVALn  Keywords   The value field shall contain a floating point number,

giving the value of the coordinate specified by the CTYPEn keyword at the

reference pixel CRPIXn.

_______________________________________________________________________________

NSDSSO FITS Standard

22_________________________________________________________SECTION_5.___HEADERS

CDELTn  Keywords   The value field shall contain a floating point number,

giving the partial derivative of the coordinate specified by the CTYPEn

keywords with respect to the pixel index, evaluated at the reference pixel,

CRPIXn, in units of the coordinate specified by the CTYPEn keyword.

CROTAn  Keywords   This keyword is used to indicate a rotation from a standard

coordinate system described by the CTYPEn to a different coordinate system in

which the values in the array are actually expressed. Rules for such rotations

are not further specified in this standard; the rotation should be explained in

comments. The value field shall contain a floating point number, giving the

rotation angle in degrees between axis n and the direction implied by the

coordinate system defined by CTYPEn.

DATAMAX  Keyword   The value field shall always contain a floating point

number, regardless of the value of BITPIX. This number shall give the maximum

valid physical value represented in the array.

DATAMIN  Keyword   The value field shall always contain a floating point

number, regardless of the value of BITPIX. This number shall give the minimum

valid physical value represented in the array.

5.2.2.6       Extension Keywords

These keywords are used to describe an extension.

EXTNAME  Keyword   The value field shall contain a character string, to be used

to distinguish among different extensions with the same value of XTENSION in a

FITS data set.

EXTVER  Keyword    The value field shall contain an integer, to be used to

distinguish among different extensions in a FITS data set with the same values

for XTENSION and EXTNAME. The values need not start with 1 for the first

extension with a particular value of EXTNAME and need not be in sequence for

subsequent values. If the EXTVER keyword is absent, the data set should be

treated as if the value were 1.

EXTLEVEL Keyword   The value field shall contain an integer, specifying the

level in a hierarchy of extension levels of the extension header containing it.

The value shall be 1 for the highest level; levels with a higher value of this

keyword shall be subordinate to levels with a lower value. If the EXTLEVEL

keyword is absent, the data set should be treated as if the value were 1.

_______________________________________________________________________________

NASA Science Data Systems Standards Office

5.2.___KEYWORDS______________________________________________________________23

5.2.2.7       ASCII Tables Keywords

In addition to the mandatory keywords defined in section 5.2.1, these keywords

may be used to describe the structure of an ASCII Tables data array. They are

optional, but if they appear within an ASCII tables extension header, they must

be used as defined in this section of this standard.

TSCALn Keywords    This keyword shall be used, along with the TZEROn keyword,

when the quantity in field n does not represent a true physical quantity. The

value field shall contain a floating point number representing the coefficient

of the linear term in the linear transformation equation below, which must be

used to compute the true physical value of the field. The default value for

this keyword is 1.0. This keyword may not be used for A-format fields.

TZEROn Keywords    This keyword shall be used, along with the TSCALn keyword,

when the quantity in field n does not represent a true physical quantity. The

value field shall contain a floating point number representing the zero point

for the true physical value of field n. The default value for this keyword is

0.0. This keyword may not be used for A-format fields.

The transformation equation used to compute a true physical value from the

quantity in field n is

physical value = TZEROn + TSCALn x field value

TNULLn  Keywords   The value field shall contain the character string that

represents an undefined value for field n. The character value should be in the

format given by the TFORMn keyword for field n.

TTYPEn  Keywords   The value field shall contain a character string, giving the

name of field n. It is recommended that only upper case letters, digits and

underscore (hexadecimal code 5F) be used in the name. The use of identical

names for different fields should be avoided.

TUNITn  Keywords   The value field shall contain a character string describing

the ultimate physical units in which the quantity in field n is expressed.

5.2.3       Additional  Keywords

5.2.3.1       Requirements

New keywords may be devised in addition to those described in this standard, so

long as they are consistent with the generalized rules for keywords and do not

_______________________________________________________________________________

NSDSSO FITS Standard

24_________________________________________________________SECTION_5.___HEADERS

conflict with mandatory or reserved keywords.

5.2.3.2       Restrictions

No keyword in the primary header shall specify the presence of a specific

extension in a FITS file; only the EXTEND keyword described in Section 5.2.1.2

shall be used to indicate the possible presence of extensions. No keyword in

either the primary or extension header shall explicitly specify the physical

block size, other than the BLOCKED keyword of Section 5.2.2.1

5.3        Value/Comment

The structure of the value field shall be determined by the type of the

variable.

5.3.1       Character  String

If the value is a character string, column 11 shall contain a single quote

(hexadecimal code 27, "'"); the string shall follow, starting in column 12,

followed by a closing single quote (also hexadecimal code 27) that should not

occur before column 20 and must occur in or before column 80. Proper

interpretation of the data set should not require decoding any more than the

first eight characters of a character string. The Character string can only be

composed of the printable ASCII characters (hexadecimal codes 20 through 7E).

5.3.2       Logical  Variable

If the value is a logical constant it shall appear as a T or F in column 30.

5.3.3       Integer

If the value is an integer, the ASCII representation shall appear right

justified in columns 11-30. For a complex integer, the imaginary part shall be

right justified in columns 31-50.

5.3.4       Real  Floating  Point  Number

If the value is a real floating point number, the ASCII representation shall

appear in columns 11-30. Letters in the exponential form shall be upper case.

The value shall be right justified, and the decimal point must appear. Note:

The full precision of 64-bit values can not be expressed as a single value

following the rules of this standard.

_______________________________________________________________________________

NASA Science Data Systems Standards Office

5.3.___VALUE/COMMENT_________________________________________________________25

5.3.5       Complex  Floating  Point  Number

5.3.5.1       Real Part

If the value is a complex floating point number, the ASCII representation of

the real part shall appear in the same manner as a real floating point number

(see above).

5.3.5.2       Imaginary Part

The ASCII representation of the imaginary part of a complex floating point

number shall appear in columns 31 - 50. Letters in the exponential form shall

be upper case. The value shall be right justified, and the decimal point must

appear. Note: The full precision of 64-bit values can not be expressed as a

single value following the rules of this standard.

_______________________________________________________________________________

NSDSSO FITS Standard

26_________________________________________________________SECTION_5.___HEADERS

_______________________________________________________________________________

NASA Science Data Systems Standards Office

_____________________________________________________________________________27

Section   6

Data   Representation

Primary and extension data shall appear in one of the formats described in this

section.

6.1        Characters

Each character shall be represented by one byte. A character shall be written

in 7-bit ASCII [8], right justified in the byte. The high-order parity bit

shall be zero.

6.2        Integers

6.2.1       Eight-bit

Eight-bit integers shall be unsigned binary integers, stored in one byte.

6.2.2       Sixteen-bit

Sixteen-bit integers shall be twos-complement signed binary integers, contained

in two bytes. The sign bit shall be the leftmost bit of the leftmost byte.

Significance shall decrease from left to right; the least significant bit shall

be the rightmost bit of the rightmost byte.

6.2.3       Thirty-two-bit

Thirty-two bit integers shall be twos-complement signed binary integers,

contained in four bytes. The sign bit shall be the leftmost bit of the leftmost

byte. Significance shall decrease from left to right; the least significant bit

shall be the rightmost bit of the rightmost byte.

_______________________________________________________________________________

NSDSSO FITS Standard

28_____________________________________________SECTION_6.___DATA_REPRESENTATION

6.3        IEEE-754  Floating  Point

Transmission of 32- and 64-bit floating point data within the FITS format will

use the ANSI/IEEE-754 standard [9]. BITPIX = -32 and BITPIX = -64 signify 32-

and 64-bit IEEE floating point numbers; the absolute value of BITPIX is used

for computing the sizes of data structures. The full IEEE set of number forms

is allowed for FITS interchange, including all special values (e.g., the

"Not-a-Number" cases). The order of the bytes will be sign and exponent first,

followed by the mantissa bytes in order of decreasing significance. The BLANK

keyword should not be used when BITPIX = -32 or -64.

6.3.1       Thirty-two-bit  Floating  Point

6.3.1.1       Interpretation

Lowest numbered (leftmost) bits are most significant. The IEEE NaN

(Not-a-Number) values shall be used to represent undefined values. All IEEE

special values are recognized.

sign    (exponent - 127)

value = (-1)     x 2                 x mantissa

6.3.1.2       Structure

Table 6.1 describes the bit structure of 32-bit floating point values.

Bit Positions        Content

__(left_to_right)_____________________

1                sign

2 - 9            exponent

10 - 32           mantissa

Table 6.1:  Content of 32-bit floating point bit positions.

_______________________________________________________________________________

NASA Science Data Systems Standards Office

6.3.___IEEE-754_FLOATING_POINT_______________________________________________29

6.3.2       Sixty-four-bit  Floating  Point

6.3.2.1       Interpretation

Lowest numbered (leftmost) bits are most significant. The IEEE NaN

(Not-a-Number) values shall be used to represent undefined values. All IEEE

special values are recognized.

sign    (exponent - 1023)

value = (-1)     x 2                  x mantissa

6.3.2.2       Structure

Table 6.2 describes the bit structure of 64-bit floating point values.

Bit Positions         Content

__(left_to_right)_____________________

1                sign

2 - 12           exponent

13 - 64           mantissa

Table 6.2:  Content of 64-bit floating point bit positions.

_______________________________________________________________________________

NSDSSO FITS Standard

30_____________________________________________SECTION_6.___DATA_REPRESENTATION

_______________________________________________________________________________

NASA Science Data Systems Standards Office

_____________________________________________________________________________31

Section   7

Data   Records

7.1        Primary  Array

The data values shall be stored as a byte stream, with no embedded fill or

blank space. The first value shall be stored in the first position of the first

array record. The first value of each subsequent row of the array shall be

stored in the position immediately following the last value of the previous

row, i.e., the data values are stored as a byte stream, with no embedded fill

or blank space. A primary array, when present, shall consist of an array of 1

to 999 dimensions. Arrays of more than one dimension shall be stored in a

sequence such that the index along axis 1 varies most rapidly, that along axis

2 next most rapidly, and those along subsequent axes progressively less

rapidly, with that along axis n varying least rapidly; i. e., the elements of

an array A(x1 ; x2 ; . . .; xm ) shall be in the order shown in Figure 7.1,

where m is the value of NAXIS. The remainder of the last data record, following

the last element of the array shall be filled as prescribed in Section 4.3.2.

7.2        ASCII  Tables  Extension

Data shall appear as an ASCII Tables extension if the primary header of the

FITS file has the keyword EXTEND set to T and the first keyword of that

extension header has XTENSION=~'TABLE~~~'.

7.2.1       Storage

The data shall be stored as a two-dimensional character array. The row length

and the number of rows shall be specified by keywords in the associated header

records. The number of characters in a row and the number of rows in the table

shall determine the size of the character array. Every row in the array shall

have the same number of characters. The first character of the first row shall

be at the start of the record immediately following the last header record.

The first character of subsequent rows shall follow immediately the character

at the end of the previous row, independent of the record structure. The

positions in the last data record after the last character of the last row of

the data array shall be filled with ASCII blanks.

_______________________________________________________________________________

NSDSSO FITS Standard

32____________________________________________________SECTION_7.___DATA_RECORDS

A(1, 1, . . ., 1),

A(2, 1, . . ., 1),

.

.

.,

A(NAXIS1, 1, . . ., 1),

A(1, 2, . . ., 1),

A(2, 2, . . ., 1),

.

.

.,

A(NAXIS1, 2, . . ., 1),

.

.

.,

A(1, NAXIS2, . . ., NAXISm),

.

.

.,

A(NAXIS1, NAXIS2, . . ., NAXISm)

Figure 7.1: Arrays of more than one dimension shall be stored in a sequence

such that the index along axis 1 varies most rapidly and those along subsequent

axes progressively less rapidly. Except for the location of the first element,

array structure is independent of record structure.

_______________________________________________________________________________

NASA Science Data Systems Standards Office

7.2.___ASCII_TABLES_EXTENSION________________________________________________33

7.2.2       Fields

Each row in the array shall consist of a sequence of fields, with one entry in

each field. For every field, the format of the stored information, location in

the row of the beginning of the field and (optionally) the field name, shall be

specified in keywords of the associated header records. A separate format

keyword must be provided for each field. The location and format of fields

shall be the same for every row.

7.2.3       Entries

All data in an ASCII tables extension record shall be 7-bit printable ASCII

characters with the eighth (parity) bit set to zero (hexadecimal codes 20

through 7E). The only possible formats shall be FORTRAN I, A, F, E, or D. If

values of -0 and +0 must be distinguished, then the sign character should be

stored in a separate field in character format. TNULL keywords may be used to

specify a character string that represents an undefined value in each field.

The characters representing an undefined value may differ from field to field

but must be the same within a field.

_______________________________________________________________________________

NSDSSO FITS Standard

34____________________________________________________SECTION_7.___DATA_RECORDS

_______________________________________________________________________________

NASA Science Data Systems Standards Office

_____________________________________________________________________________35

Section   8

Restrictions   on   Changes

Any structure that is a valid FITS structure shall remain a valid FITS

structure at all future times. Use of certain valid FITS structures may be

deprecated by this or future FITS standard documents.

_______________________________________________________________________________

NSDSSO FITS Standard

36_________________________________________SECTION_8.___RESTRICTIONS_ON_CHANGES

_______________________________________________________________________________

NASA Science Data Systems Standards Office

_____________________________________________________________________________37

Appendix   A

Random   Groups   Structure

(This Appendix is a part of the NSDSSO FITS Standard)

Although it is part of the FITS standard, the random groups structure for

the primary data has been used almost exclusively for applications in radio

interferometry; outside this field, few FITS readers can read data in random

groups format. The evolving binary tables format will eventually be able to

accommodate the structure described by random groups. While existing FITS data

sets use the format, and it is therefore included in this standard, its use for

future applications is deprecated by this document.

A.1         Keywords

A.1.1        Mandatory  Keywords

If the primary data are in Random Groups format, the card images of the primary

header must use the following keywords in the order given here.

1.  SIMPLE

2.  BITPIX

3.  NAXIS

3.  + n ( n=1, . . ., value of NAXIS ). NAXISn

.

.

.

(other keywords, including)

GROUPS

PCOUNT

GCOUNT

.

.

.

last   END

_______________________________________________________________________________

NSDSSO FITS Standard

38________________________________________APPENDIX_A.___RANDOM_GROUPS_STRUCTURE

A.1.1.1        SIMPLE Keyword

The card image containing this keyword is structured in the same way as for a

simple array (Section 5.2.1).

A.1.1.2        BITPIX Keyword

The card image containing this keyword is structured in the same way as for a

simple array (Section 5.2.1).

A.1.1.3        NAXIS Keyword

The value field shall contain an integer ranging from 1 to 999, representing

one more than the number of axes in each data array.

A.1.1.4        NAXIS1 Keyword

The value field shall contain the integer 0, as a signature of random groups

format.

A.1.1.5        NAXISn Keywords (n=2, . . ., value of NAXIS)

The value field shall contain an integer, representing the number of positions

along axis n-1 of each ordinary data array.

A.1.1.6        GROUPS Keyword

The value field shall contain the logical constant T. The value T associated

with this keyword implies that the primary array is in random groups format.

A.1.1.7        PCOUNT Keyword

The value field shall contain an integer equal to the number of parameters

preceding each group.

_______________________________________________________________________________

NASA Science Data Systems Standards Office

A.2.___DATA_STORAGE__________________________________________________________39

A.1.1.8        GCOUNT Keyword

The value field shall contain an integer equal to the number of random groups

in the data array.

A.1.1.9        END Keyword

The card image containing this keyword is structured in the same way as for a

simple array (Section 5.2.1).

A.1.2        Reserved  Keywords

A.1.2.1        PTYPEn Keywords

The value field shall contain a character string, describing the physical

quantity or coordinate represented by the value for parameter n, as derived

from the quantity stored in the FITS data set and the PSCALn and PZEROn

keywords. If the PTYPEn keywords for more than one value of n have the same

associated value, then the data value for the physical quantity or coordinate

so defined is to be obtained by adding the derived data values of the

corresponding parameters.

A.1.2.2        PSCALn Keywords

This keyword shall be used, along with the PZEROn keyword, when the data set

parameter values are not the true physical values, to transform the data set

parameter values to the true physical values they represent, using the linear

transformation equation below with the PZEROn keyword. The value field shall

contain a floating point number representing the coefficient of the linear term

in the scaling equation, the scaling factor between true values and data set

parameter values at zero offset.

A.1.2.3 PZEROn Keywords

This keyword shall be used, along with the PSCALn keyword, when the data set

param- eter values are not the true physical values, to transform data set

parameter values to the true values. The value field shall contain a floating

_______________________________________________________________________________

NSDSSO FITS Standard

40________________________________________APPENDIX_A.___RANDOM_GROUPS_STRUCTURE

point number, representing the true value corresponding to a data set parameter

value of zero. The transformation equation is as follows:

physical value = PZEROn + PSCALn x data set value

A.2         Data  Storage

Values shall be stored as a series of groups. The number of groups shall be

specified by the GCOUNT keyword in the associated header record. Each group

shall consist of the number of parameters specified by the PCOUNT keyword

followed by an array. The first parameter of the first group shall appear in

the first location of the first data record. The first element of each array

shall immediately follow the last parameter associated with that group. The

first parameter of any subsequent group shall immediately follow the last

member of the array of the previous group. The arrays shall be organized

internally in the same way as an ordinary primary array.

A.3         Data  Representation

Permissible data representations are the same as those for an ordinary primary

array. Parameters and members of associated data arrays shall have the same

representation. Should more precision be required for an associated parameter

than for a member of a data array, the parameter shall be divided into two

addends, represented by the same value for the PTYPEn keyword. The value shall

be the sum of the physical values, which may have been obtained from the array

values using the PSCALn and PZEROn keywords.

_______________________________________________________________________________

NASA Science Data Systems Standards Office

_____________________________________________________________________________41

Appendix   B

A   Binary   Table   Extension

(This Appendix is not part of the NSDSSO FITS Standard but is included for

informational purposes only.)

The new "3D"[1] binary tables format is under consideration by the

regional and international FITS committees. Its present form is expected to be

a subset of the eventual structure that is adopted; some structures and

capabilities remain to be added. It is in use at a number of installations, has

been implemented under AIPS and exchanged between AIPS and MIDAS, but much of

the existing FITS-reading software yet cannot decode the format. Because it is

becoming widely used, and because it illustrates an application of the rules

for conforming extensions, it is described in this Appendix.

B.1         Identification

The interim identification of a binary table is indicated if the primary header

of the FITS file has the keyword EXTEND set to T and the first keyword of the

associated extension header has XTENSION= 'A3DTABLE'.

B.2         Header

The header of a binary table extension shall conform to the requirements for

FITS headers specified in Section 5.

B.2.1        Required  Keywords

The card images in the header of a binary table extension use the following

keywords in the order given in Table B.1.

_________________________________________________

[1] The term "3D" derives from the capacity to store vectors as fields in

binary tables. Thus, the table effectively has depth.

_______________________________________________________________________________

NSDSSO FITS Standard

42_______________________________________APPENDIX_B.___A_BINARY_TABLE_EXTENSION

1      XTENSION

2      BITPIX

3      NAXIS

4      NAXIS1

5      NAXIS2

6      PCOUNT

7      GCOUNT

8      TFIELDS

.

.

(other keywords, which must include)

TFORMn, n = 1, 2, . . ., TFIELDS

.

.

last    END

Table B.1:  Required keywords in the interim binary table extension.

B.2.1.1       XTENSION Keyword

The value field shall contain the character string 'A3DTABLE'.

B.2.1.2       BITPIX Keyword

The value field shall contain the integer 8, indicating that row size is given

in 8-bit bytes.

B.2.1.3       NAXIS Keyword

The value field shall contain the integer 2, indicating two axes, rows and

columns.

B.2.1.4       NAXIS1 Keyword

The value field shall contain an integer, representing the width of the table

in 8-bit bytes.

B.2.1.5       NAXIS2 Keyword

The value field shall contain an integer of value 1 or greater, representing

the number of rows in the table.

_______________________________________________________________________________

NASA Science Data Systems Standards Office

B.2.___HEADER________________________________________________________________43

B.2.1.6        PCOUNT Keyword

The value field shall contain the integer 0; no data values precede the table.

B.2.1.7        GCOUNT Keyword

The value field shall contain the integer 1; the data records contain a single

table.

B.2.1.8        TFIELDS Keyword

The value field contains an integer representing the number of fields in each

row.

B.2.1.9        TFORMn Keywords

The value field contains a character string describing the data type of field

n. The following values are permitted, each representing the data type

indicated:

_Value_field________Data_Type_____________________

rL                Logical

rX                Bit array

rA                ASCII characters

rI                16-bit integer

rJ                32-bit integer

rE                32-bit floating point

rD                64-bit floating point

Table B.2:  Valid TFORMn format values in the interim binary table extension.

The lower case r preceding the upper case type indicator is the repeat

count, describing how many times the data type appears within field n. A

repeat count of zero indicates that the entry defined in the header is omitted

in the table.

B.2.1.10         END Keyword

This keyword has no associated value.  Columns 9-80 shall be blank.

_______________________________________________________________________________

NSDSSO FITS Standard

44_______________________________________APPENDIX_B.___A_BINARY_TABLE_EXTENSION

B.2.2        Reserved  Keywords

These keywords are optional but are used in A 3-D Tables extensions only in the

manner defined in this document.

B.2.2.1       TTYPEn Keyword

The value field contains a character string representing the label or heading

for field n.

B.2.2.2       TUNITn Keywords

The value field contains a character string representing the physical units of

the quantities contained in field n.

B.2.2.3       TNULL Keyword.

The value field contains an integer representing the value used in integer

fields to represent an undefined value. The IEEE NaN (Not-a-Number) is used

for the same purpose in floating point fields.

B.3         Storage

Data are stored as a two dimensional array. The number of bytes in a row and

the number of rows are specified by the NAXIS1 and NAXIS2 keywords in the

associated extension header. Every row in the array must have the same number

of bytes. The number of bytes in a row and the number of rows in the table

determines the size of the array. The first byte of the first row is at the

start of the record immediately following the last header record. The first

byte of every subsequent row immediately follows the last byte of the row

preceding it.

B.4         Format

Each row in a binary table extension array consists of a sequence of fields,

each of which consists of zero or more entries. The location, length and format

of each field are the same for every row. The field sequence is specified by

the TFORMn keywords of the associated header. All entries in a field must be of

the same data type. For each field, the number of entries, data type, and

(optionally) the field name, are specified in keywords of the associated header

records. A separate format keyword must be provided for each field.

_______________________________________________________________________________

NASA Science Data Systems Standards Office

B.5.___DATA_TYPES____________________________________________________________45

B.5         Data  Types

Each field in the table contains a binary representation of information in one

of the seven following forms:

B.5.1        Logical

A logical value consists of an 8-bit ASCII T for true or F for false.

B.5.2        Bit  Arrays

A bit array consists of an integral number of bytes with trailing bits zero. A

bit array shall start in the most significant bit of the first byte; bits will

follow in order of decreasing significance

B.5.3        ASCII  Characters

Each character shall be represented by one byte. A character shall be written

in 7-bit ASCII [8], right justified in the byte. The high-order parity bit

shall be zero.

B.5.4        Eight-bit  Integers

Eight-bit integers shall be unsigned binary integers, stored in one byte.

B.5.5        Sixteen-bit  Integers

Sixteen-bit integers shall be twos-complement signed binary integers, contained

in two bytes. The sign bit shall be the leftmost bit of the leftmost byte.

Significance shall decrease from left to right; the least significant bit shall

be the rightmost bit of the rightmost byte. The TNULL keyword is used if needed

to specify the symbol corresponding to an undefined value.

B.5.6        Thirty-two-bit  Integers

Thirty-two bit integers shall be twos-complement signed binary integers,

contained in four bytes. The sign bit shall be the leftmost bit of the leftmost

byte. Significance shall decrease from left to right; the least significant bit

shall be the rightmost bit of the rightmost byte. The TNULL keyword is used if

needed to specify the symbol corresponding to an undefined value.

_______________________________________________________________________________

NSDSSO FITS Standard

46_______________________________________APPENDIX_B.___A_BINARY_TABLE_EXTENSION

B.5.7        Thirty-two-bit  Floating  Point

B.5.7.1        Interpretation

Lowest numbered (leftmost) bits are most significant. The IEEE NaN

(Not-a-Number) values shall be used to represent undefined values. All IEEE

special values are recognized.

sign    (exponent - 127)

value = (-1)     x 2                 x mantissa

B.5.7.2       Structure

Table 6.1 describes the bit structure of 32-bit floating point values.

B.5.8       Sixty-four-bit  Floating  Point

B.5.8.1       Interpretation

Lowest numbered (leftmost) bits are most significant. The IEEE NaN

(Not-a-Number) values shall be used to represent undefined values. All IEEE

special values are recognized.

sign    (exponent - 1023)

value = (-1)     x 2                  x mantissa

B.5.8.2       Structure

Table 6.2 describes the bit structure of 64-bit floating point values.

_______________________________________________________________________________

NASA Science Data Systems Standards Office

_____________________________________________________________________________47

Appendix   C

Implementation   on   Physical   Media

(This Appendix is not part of the NSDSSO FITS Standard, but is included as a

guide to recommended practices)

C.1         Block  Size

The block size (physical record length) for transport of data should, where

possible, equal the logical record length or an integer blocking factor times

this record length. Standard values of the blocking factor may be specified for

each medium; if not otherwise specified, the expected value is unity.

C.1.1        Magnetic  Tape

For nine-track magnetic tapes conforming to the ANSI X3.40-1983 specifications

[10], there should be from one to 10 logical records per physical block. The

BLOCKED keyword (section 5.2.2.1) may be used to warn that there may be more

than one logical record per physical block. The last physical block of a FITS

file should be truncated to the minimum number of FITS logical records required

to hold the remaining data, in accordance with ANSI X3.27-1978 specifications

[11]. With the issuance of this standard the BLOCKED keyword is deprecated by

this document.

C.1.2        Other  Media

For media where the physical block size cannot be equal to or an integral

multiple of the FITS logical record length of 23040-bits (2880 8-bit bytes),

records should be written over multiple blocks. Conventions regarding the

relation between physical block size and logical record length of FITS data

sets have not been otherwise established for other media.

_______________________________________________________________________________

NSDSSO FITS Standard

48_______________________________APPENDIX_C.___IMPLEMENTATION_ON_PHYSICAL_MEDIA

C.2         Physical  Properties  of  Media

The arrangement of digital bits and other physical properties of any medium

should be in conformance with the ANSI standard for that medium as specified in

the relevant document.

C.3         Labeling

C.3.1       Tape

Tapes may be either ANSI standard labeled or unlabeled. Unlabeled tapes are

preferred.

C.3.2       Other  Media

Conventions regarding labels for FITS data sets have not been established for

other media.

C.4         FITS  File  Boundaries

C.4.1       Magnetic  Tape

Individual FITS files are terminated by a tape-mark.

C.4.2       Other  Media

For media where the physical record size cannot be equal to or an integral

multiple of the standard FITS logical record length, a record of fewer than

23040 bits (2880 8-bit bytes) immediately following the end of the primary

header, data, or an extension should be treated as an end-of-file. Otherwise,

individual FITS data sets should be terminated by a delimiter appropriate to

the medium, analogous to the tape end-of-file mark. If more than one FITS data

set appears on a physical structure, the appropriate end-of-file indicator

should immediately precede the start of the primary headers of all data sets

after the first. FITS data sets should start at the start of a file.

C.5         Multiple  Physical  Volumes

Storage of a single FITS file on more than one tape or on multiple units of any

other medium is not supported.

_______________________________________________________________________________

NASA Science Data Systems Standards Office

_____________________________________________________________________________49

Appendix   D

Summary   of   Keywords

(This Appendix is not part of the NSDSSO FITS Standard, but is included for

convenient reference).

Principal    Conforming    ASCII Table     Random Groups   Binary Table

___HDU________Extension______Extension_________Extension_____Extension_____

SIMPLE      XTENSION      XTENSION [1]    SIMPLE           XTENSION [2]

BITPIX      BITPIX        BITPIX  =  8    BITPIX           BITPIX  =  8

NAXIS       NAXIS         NAXIS  =  2     NAXIS            NAXIS  =  2

NAXISn      NAXISn        NAXIS1          NAXIS1  =  0     NAXIS1

EXTEND      PCOUNT        NAXIS2          NAXISn           NAXIS2

END         GCOUNT        PCOUNT  =  0    GROUPS           PCOUNT  =  0

END           GCOUNT  =  1    PCOUNT           GCOUNT  =  1

TFIELDS         GCOUNT           TFIELDS

TBCOLn          END              TFORMn

TFORMn                           END

___________________________END______________________________________________

[1] XTENSION= 'TABLE   ' for the ASCII Table extension.

[2] XTENSION= 'A3DTABLE' for the interim binary table extension.

Table D.1: Mandatory FITS keywords for the structures described in this

document.

_______________________________________________________________________________

NSDSSO FITS Standard

50____________________________________________APPENDIX_D.___SUMMARY_OF_KEYWORDS

Principal HDU      Conforming    ASCII Table   Random Groups  Binary Table

_General____Array____Extension_____Extension_____Extension______Extension___

DATE       BSCALE    EXTNAME       TSCALn        PTYPEn         TTYPEn

ORIGIN     BZERO     EXTVER        TZEROn        PSCALn         TUNITn

BLOCKED    BUNIT     EXTLEVEL      TNULLn        PZEROn         TNULL

AUTHOR     BLANK                   TTYPEn

REFERENC   CTYPEn                  TUNITn

COMMENT    CRPIXn

HISTORY    CROTAn

CRVALn

DATE-OBS   CDELTn

TELESCOP   DATAMAX

INSTRUME   DATAMIN

OBSERVER

OBJECT

EQUINOX

EPOCH_______________________________________________________________________

Table D.2: Reserved FITS keywords for the structures described in this

document. Note that the EPOCH and BLOCKED keywords are deprecated by this

document.

__Production___Bibliographic___Commentary___Observation_____Array___

DATE         AUTHOR          COMMENT      DATE-OBS      BSCALE

ORIGIN       REFERENC        HISTORY      TELESCOP      BZERO

BLOCKED                                   INSTRUME      BUNIT

OBSERVER      BLANK

OBJECT        CTYPEn

EQUINOX       CRPIXn

EPOCH         CROTAn

CRVALn

CDELTn

DATAMAX

__________________________________________________________DATAMIN___

Table D.3: General Reserved FITS keywords described in this document. Note that

the EPOCH and BLOCKED keywords are deprecated by this document.

_______________________________________________________________________________

NASA Science Data Systems Standards Office

_____________________________________________________________________________51

Appendix   E

NSDSSO   Publications

_____Document__________Title______________Date___________Status_____

NSDSSO 100-0.1     FITS Standard     December, 1990   Draft Standard

____________________________________________________________________

Table E.1:  NSDSSO Publications

_______________________________________________________________________________

NSDSSO FITS Standard

#